INDUSTRIAL ALUMINIUM

Description:	An Aluminium based Alkyd paint.

For Use On:	Roofing, metals and timber

Appearance:	Metallic sheen

Application:	Spray or brush.  Product should be thoroughly stirred for at least 5 minutes before use.  A high speed mixer is required for mixing

Dry Time:	60 minutes at 20oC and 50% relative humidity.

Thinner:	Mineral Turpentine

Clean Up:	Mineral Turpentine

Colour Range:	Silver

Container Sizes:	1 Litre, 4 Litre, 10 litre & 20 Litre

Coverage:	15-18m2 per litre

Surface Preparation:	As with any product, proper surface preparation is essential for a successful application. We emphasise cleanliness on old and new surfaces before paint application, as this is very important to ensure good bonding and lasting quality. Degreasing may be required.

a)  	Make sure all surfaces are Clean and Dry.

b)  	Dirt and Grime: Wash and scrub with water and wash again. Leave to dry.

- Oily substracts: Use suitable solvent eg: Reidpaint Medium Thinners. Wash and scrub with water and wash again.  Leave to dry.

Coating Format:	Substrate to be between 10  - 25 degrees.

Recommend 1-2 coats.

Recoat after 2-2½ hours dependant on temperature.

Flammability in Can:	Flammable

General Precautions:	No protective clothing necessary but good ventilation required

* * * * * * * * * * * * * * * *

IN the interest of personal safety Reidpaints Ltd recommend all paint products be stored securely away from children.

REIDPAINTS  Ltd cares about the environment and our valued customers, if you have any questions on the product you are using from application to disposal, or require Material Safety Handling information please contact Reidpaints free phone (0800) 80 PAINT (72468).

THE information stated on this Data Sheet, to the best of our knowledge is true and accurate.   Any recommendations or suggestions are given in good faith but without guarantee since conditions of use and application are beyond Reidpaints Ltd control.  Some guarantees may apply by law.